ECE 528/L

Robotics and Embedded Systems with Lab

Final Project

Heat-Seeking Robot

Suren Bursalyan

Aramayis Sargsyan

Instructor: Aaron Nanas

Fall 2025

Introduction

The main objective of our final project, the heat-seeking robot was to create a robot that scans for a heat source. Once found, the robot would lock on and follow its target. Should the robot lose tracking of its target, it will stop and return to scanning for a new target. This project will serve to showcase core robotics concepts, such as interfacing with peripherals, interfacing with hardware, communication between hardware and a microcontroller using the I2C protocol, generating PWM signals, GPIO, sensor fusion, and decision-making algorithms.

Components Used

| Component | Brand | Model | Purpose |
| --- | --- | --- | --- |
| Microcontroller | Elegoo | Uno R3 | Hardware Control |
| Tank Conqueror Kit | Elegoo | N/A | Chassis |
| Servo Motor | Elegoo | SG-90 | Pan IR Camera Left/Right |
| Servo Motor | Elegoo | SG-90 | Tilt IR Camera Up/Down |
| Stepper Motor | Elegoo | N/A | Drive Left Track |
| Stepper Motor | Elegoo | N/A | Drive Right Track |
| IR Thermal Camera Array | Adafruit | AMG8833 | Heat Sensing |
| Stepper Motor Driver | BOJACK | L298N | Supply 5V to Servo/Stepper Motors (High Current) |
| Arduino External Power Supply x2 (w/ 9V Battery Adapter) | Elegoo | N/A | Enable Powering of Microcontroller via 9V Batteries |
| 9V Battery x2 | Amazon Basics | N/A | Power Supply |

Analysis and Results

The result of this final project was overall a success. We managed to create and implement a heat-seeking robot meeting all of the design criteria. We did need an extra power source (the motor driver serves as a power source) since all the hardware that was required needed 5 volts to operate. The power source that the kit came with was faulty/could not power everything. Furthermore, the power source that was provided in the kit could not implement the logic required for the motors in order to control movement and direction of the robot, rather, it only powered the motors to move forward. In addition to these challenges, a new communication protocol - I2C was learned.

The advantage of using I2C over other communication protocols, especially in our project is that it allowed us to read all 64 infrared sensors in our thermal camera array using only 2 wires rather than 64. Namely, ‘SCL’ and ‘SDA’ where ‘SCL’ is short for “Serial Clock” and ‘SDA’ is short for “Serial Data”. Serial Clock is an output from the microcontroller to the thermal camera array in the form of a square pulse which essentially requests data from the thermal camera array. This is because in the I2C protocol, sensors are not constantly transmitting data, but rather only transmit their current data upon request from the microcontroller. The second wire, ‘SDA’ is short for “Serial Data” and is the actual data containing the temperatures in degrees Celsius that is sent from the thermal camera array to the microcontroller after a request from the microcontroller is received.

Conclusion

Overall, this project taught us many different things about robotics. The biggest takeaway was of course learning the I2C protocol, but the challenges faced while programming the logic behind movement of the robot, namely controlling the movement of the motors from scratch on a completely different microcontroller than the one used throughout the labs, proved to be valuable lessons.

GitHub Repository Link

https://github.com/surenb1/Heat-Seeking-Robot-Arduino