| Component | Brand | Model | Purpose |
| --- | --- | --- | --- |
| Microcontroller | Elegoo | Uno R3 | Hardware Control |
| Tank Conqueror Kit | Elegoo | N/A | Chassis |
| Servo Motor | Elegoo | SG-90 | Pan IR Camera Left/Right |
| Servo Motor | Elegoo | SG-90 | Tilt IR Camera Up/Down |
| Stepper Motor | Elegoo | N/A | Drive Left Track |
| Stepper Motor | Elegoo | N/A | Drive Right Track |
| IR Thermal Camera Array | Adafruit | AMG8833 | Heat Sensing |
| Stepper Motor Driver | BOJACK | L298N | Supply 5V to Servo/Stepper Motors (High Current) |
| Arduino External Power Supply (9V Battery Adapter) | Elegoo | N/A | Enable Powering of Microcontroller via 9V Batteries |
| 9V Battery x2 | Amazon Basics | N/A | Power Supply |

| From Pin (On Motor Driver) | To Pin | Purpose |
| --- | --- | --- |
| ENA | D5 (Arduino) | Motor A Speed Control |
| ENB | D6 (Arduino) | Motor B Speed Control |
| IN1 | D4 (Arduino) | Direction Control from Arduino |
| IN2 | D4 (Arduino) | Direction Control from Arduino |
| IN3 | D7 (Arduino) | Direction Control from Arduino |
| IN4 | D8 (Arduino) | Direction Control from Arduino |
| OUT1 | Motor A (Red) | Direction Control Output |
| OUT2 | Motor A (Black) | Direction Control Output |
| OUT3 | Motor B (Black) | Direction Control Output |
| OUT4 | Motor B (Red) | Direction Control Output |
| 12V | Power Supply Input (+) | Supply to Motor Driver |
| GND | Power Supply Input (-) | Supply to Motor Driver |
| 5V | Servo VCC (Tilt & Pan) | 5V Output from Motor Driver |